Final Project

Matt Tranchina

Computer Science, Southern New Hampshire University

CS-350: Emerging Systems Architecture and Technology

Goran Trajkovski

February 22nd, 2026

SysTec is in the process of developing a thermostat that sends data to its server’s software over Wi-Fi, and a working low-level prototype has been accomplished. The prototype uses a Raspberry Pi 4 module to run the state machine Python code that implements the use of the AHT20 temperature sensor via an I2C connection along with a universal asynchronous receiver-transmitter (UART) to send and receive data. UART is a communication protocol commonly used for serial communication between devices. When combined with Wi-Fi technology, UART Wi-Fi allows devices to transmit data wirelessly, enabling seamless communication over a local network or even the internet (Proculustech, 2025). When utilizing an integrated Wi-Fi solution, the next phase is to decide the hardware architectures of the prototype. Considering Raspberry Pi has been used for the prototype, this will be considered along with Microchip and Freescale. Key factors of the architecture include that the thermostat must support the peripherals used within the project, must connect to the cloud via Wi-Fi, and the architecture must have enough Flash and Ram to support the code. So let us compare all three architectures against these requirements.

|  | Raspberry Pi 4 Model B | Microchip PIC32WM-BW-1 | Freescale FRDM-MCXA174 |
| --- | --- | --- | --- |
| Peripherals used in project (I2C, UART, GPIO) | All compatible | All compatible | All compatible |
| LCD display | Supported via Python libraries | Supported via interactive GUI to configure files in binary and JSON formats | Supported via SmartDMA/LCD header |
| Processor | Broadcom BCM2711, quad-core Cortex-A72 (ARM v8) 64-bit SoC @ 1.8GHz | 64 MHz ARM® Cortex® | MCX A174 Arm Cortex-M33 core up to 180 MHz |
| Wi-Fi capabilities | 2.4 GHz and 5.0 GHz IEEE 802.11ac wireless, Bluetooth 5.0, BLE Gigabit Ethernet | IEEE® 802.11 b/g/n, Single Stream (1x1) 20 MHz Bandwidth WLAN Link | Not built-in, but available via Arduini, PMOD/MikroBUS modules |
| Flash | MicroSD card slot for loading operating system and data storage | On-board 1MB Flash memory, expandable via external memory interface | Up to 256 KB on-board Flash memory, expandable via external memory interface |
| Ram | 1 GB, 2GB, 4GB, 8GB | 128 KB | 64 KB |
| Operating Systems | Raspberry Pi OS, Ubuntu, Apertis, Linux, Bass, Armbian, DietPi | Typically programmed without an OS, but can use FreeRTOS | Zephyr OS |
| Languages Supported | Python, C, C++, Java, Scratch, Node.js, Rust, Go, PHP | C, C++ | C |
| Price | $38.00 – 115.00 | $6.61 | $20.00 |

From a price standpoint, the Raspberry Pi is the most expensive, but there are a lot of features and ease of mind that come with this price. While all devices support the peripherals of the project, including LCD displays, Wi-Fi capabilities, Flash and Ram, Raspberry Pi is by far the most user friendly of the three. It supports Ubuntu and Python, so there is no need to learn new IDEs or convert the already existing Python code into C or C++. The built-in Wi-Fi feature allows connection to the cloud without external antennas or modules. Not to mention the prototype was already built using the Raspberry Pi 4 module, so no additional compromises or configurations would be needed. While the Microchip is a cost-effective option for mass production, they are typically shipped without an existing operating system and can not support what the Raspberry Pi can. The Freescale microcontroller, however, can come with the Zephyr operating system, which comes with its own IDE and support center, but only supports the C language. Considering where this project currently is in development, and given the existing prototype, I would recommend the continuation of the Raspberry Pi 4 Module for the next stage of development in connecting the thermostat to the cloud.

References

Proculustech. (2025, October 27). Integrating UART WIFI in everyday smart living. Proculus Technologies Co., Ltd. https://www.proculustech.com/integrating-uart-wifi-in-everyday-smart-living#:~:text=UART%20(Universal%20Asynchronous%20Receiver%2DTransmitter,network%20or%20even%20the%20internet.

Raspberry pi 4 model B specifications – raspberry pi. (n.d.). https://www.raspberrypi.com/products/raspberry-pi-4-model-b/specifications/

PIC32MZ W1 and WFI32E01 Family Data sheet. (n.d.). https://ww1.microchip.com/downloads/aemDocuments/documents/WSG/ProductDocuments/DataSheets/PIC32MZ-W1-and-WFI32E01-Family-Data-Sheet-DS70005425.pdf

FRDM-MCXA174 Development Board using MCUXpresso. FRDM-MCXA174 Development Board using MCUXpresso | NXP Semiconductors. (2026, January 4). https://www.nxp.com/design/design-center/development-boards-and-designs/FRDM-MCXA174